SiI PCI-680 ‘base driver’ Version 1.0.0.12 Release.

Silicon Image

9501 Jeronimo Road 		Voice:	(949) 454-0800

Irvine, CA. 92618-2032		FAX:		(949) 454-8314

Web Site:		http://www.siliconimage.com

# Contents

I.	Overview

II.	Applicable Hardware

III.	Installation

IV.	New Features and Enhancements

V.	Corrections

VI.	Known Restrictions or Limitations

- Overview

This document describes the 1.0.0.12 release of the non-raid driver that supports the SiI-680 controller on most Windows operating systems.  The supported operating systems are Windows 98/ME, Windows NT 4.0, Windows 2000, and Windows XP.  The table below describes all the files involved in the release.

| #### File Name | #### Description | #### Version | #### File Date | #### File Size |
| --- | --- | --- | --- | --- |
| PnP680.sys | Driver installed on Windows NT 4.0, Windows 2000, and Windows XP systems. | 1.0.0.12 | Friday, March 15, 2002, 5:09:42 PM | 37,031 bytes |
| PnP680.mpd | Driver installed on Windows 98, and Windows ME systems. | 1.0.0.12 | Friday, March 15, 2002, 5:09:42 PM | 37,031 bytes |
| SI680.inf | File used to install driver on all supported operating systems, except Windows NT. | N/A | Friday, March 15, 2002, 5:02:14 PM | 3,493 bytes |
| SilSupp.cpl | File containing controller information control panel applet used on NT systems, and device manager property pages used on Windows 2000/XP systems. | 1.0.0.2 | Monday, December 03, 2001, 3:59:06 PM | 77,824 bytes |
| SiiSupp.vxd | Support driver used on Windows 98/ME systems to enable power management and to assist in the display of device properties. | 1.0.0.9 | Friday, March 15, 2002, 11:45:10 AM | 20,048 bytes |
| TxtSetup.oem | File used to install driver as part of the Windows NT, Windows 2000, or Windows XP operating system installation. | N/A | Tuesday, February 26, 2002, 4:44:22 PM | 633 bytes) |
| Readme.txt | File that describes how to install the driver. | N/A | Thursday, March 07, 2002, 11:47:38 AM | 13,822 bytes |

- Applicable Hardware

This release supports the following hardware:

CMD PCI-680 Revision 0

CMD PCI-680 Revision 1

CMD PCI-680 Revision 2

- Installation

Follow the enclosed README file for driver installation/update procedures.

- New Features and Enhancements

None.

- Corrections

Fixed problem on Win2K/XP of converting a drive that requires 48-bit address to a dynamic disk.

- Known Restrictions or Limitations

- To be able to use the control panel applet to view device properties on Windows 98/ME systems, the SiiSupp.vxd requires that the first device on the 680 controller not be CD/DVD rom type device.  In other words, the driver must detect a hard disk device before detecting the CD/DVD rom device.  The driver scans for devices in the following order; primary master, primary slave, secondary master, secondary slave.  If its not desirable to re-address devices attached to the SiI-680 controller, ensuring that there is a CD loaded in the CD/DVD rom will also enable the Vxd to work correctly.

- After upgrading the generic Windows ME driver for the motherboard ATA controller with the driver provided by Intel, the system may no longer be able to go into standby mode.  When an attempt is made to place the system into standby a message will be displayed saying that the SiI-680 is unable to go into a low power mode.  There appears to be some conflict between the power management Vxd installed with the Intel driver and the Vxd installed with the SiI-680 driver.  At this time there is no workaround for this problem except to avoid updating the Intel driver.

- (SCR# 1035) On the Intel D815EEA/EEA2 platform with Windows 98 second edition installed, the error message “Write fault error writing to device AUX, Abort, Retry, Ignore, Fail?” appears during the OS boot.  This happens if the onboard Intel IDE driver (82801BA chip set) is updated with version 6.03.009 or version 6.20.  Both versions will cause the write fault error if both the hard disk and CD-ROM drives are connected to the SiI-680 controller after the driver update.  There are two possible workarounds:

  - Uninstall the Intel driver.

  - Locate the ATA/ATAPI devices such that at least one device is attached to the onboard controller.

      - (SCR# 1036) Adaptec Easy CD Creator version 5.0 may cause a Windows 98/ME system to bugcheck with a “Windows protection error”.  The bugcheck seems to only occur when Easy CD Creator is run for the 1st time and there is a CD/DVD-Rom attached to the motherboard ATA controller, and a CD-RW device attached to the 680 controller.  There are two known workarounds for this problem:

- Attach all ATAPI devices to either the motherboard ATA controller or to the 680 controller.

- Move any CD/DVD-Roms that are attached to the motherboard ATA controller to the 680 controller.